Name: _____Servando_Olvera________ ID# ________1001909287_______ Date Submitted: _____03-19-2024____ Time Submitted _____4:00_pm______ CSE 3341 Digital Logic Design II CSE 5357 Advanced Digital Logic Design Spring Semester 2024 Lab 4 – Registered Eight x Eight Signed Multiplier 200 points Due Date – March 21, 2024, 11:59 PM Submit on Canvas Assignments Note – Late submissions will not be accepted!

DESIGN REQUIREMENTS – SIGNED VERSION

Your assignment is to design a registered eight-by-eight signed multiplier by enhancing the four- by-four unsigned shift-and-add multiplier shown below and discussed in class.

REQUIREMENTS:

1. Registered 8 x 8 multiplier
2. Signed numbers (use 2’s complement for negative numbers)
3. SystemVerilog implementation
4. Design verification (simulation)
5. DE10-Lite realization

DESIGN VERIFICATION (simulation)

1. Simulate your designs to verify their correctness. Use the following values for M and Q in
your simulations.

(a) 01111111 x 00000001
(b) 00010101 x 00101010
(b) 01111111 x 11111111
(c) 10101010 x 00110011
(d) 10101010 x 11111110
(e) 11011101 x 11001101

2. Include a screen shot of your simulation waveforms in your report.
3. Record the simulation results in a table for your report (use hexadecimal)
4. How many clock cycles does it take for each case to complete?

RTL ANALYSIS

1. Generate RTL diagrams using the Quartus Prime Netlist Viewer for each version.
2. Record the compilation summary for your report. How many ALM, registers, and pins does
your design require?

TIMING ANALYSIS

1. Run a timing analysis on your signed multiplier and determine its maximum operating
speed in GHz.

2. Capture a screen shot of your timing analysis waveform showing the fastest clock speed
your design will accommodate.

DE10-Lite IMPLEMENTATION (signed version only)

1. Implement your signed multiplier on the DE10-Lite using the following inputs/outputs. Use

pin assignments of your choice

Inputs M, Q, InM, InQ,

Outputs Mout, Qout, Pout. Display in hexadecimal on the HEX displays.

2. Include a table of your pin assignments in your report.
3. Program the DE10-Lite with your design.

DATA PATH DIAGRAM

CONTROL PATH DIAGRAM

SYSTEM-VERILOG CODE

Top Module

Register Module

Multiplier Module

Multiplier Control Module

MUX Controller Module

Finite State Machine Module

MUX/ Four to One Decoder Module

Clock Ladder Module

Binary to Seven-Seg Display Decoder Module

SIMULATION RESULTS WAVEFORM

PIN ASSIGMENTS

RTL DIAGRAMS

Top Module

Registers A & B

Multiplier

Multiplier Control

Clock Ladder

Product Register

MUX/Controller

Four to One Decoder

Binary To Seven Segment Display Decoder (Product)

Finite State Machine

Binary To Seven Segment Display Decoder

(Used 4 times to properly display Mout & Qout)

Four to One Decoder

# of ALMs 	= 165

# of REGISTERS 	= 92

# of PINS		= 68

TIMING ANALYSIS DIAGRAMS

Max Operating Speed: ~ 0.19 GHz

Fastest Clock Speed: 5.3 ns

TEST RESULTS

Signed Tests

| Test | M x Q (Binary) | M x Q (Hex) | Product (Hex) | Clock Cycles |
| --- | --- | --- | --- | --- |
| a) | 0111 1111 x 0000 0001 | 7F x 01 | 007F | 20 |
| b) | 0001 0101 x 0010 1010 | 15 x 2A | 0372 | 22 |
| c) | 0111 1111 x 1111 1111 | 7F x FF | FF81 | 27 |
| d) | 1010 1010 x 0011 0011 | AA x 33 | EEDE | 23 |
| e) | 1010 1010 x 1111 1110 | AA x FE | 00AC | 26 |
| f) | 1101 1101 x 1100 1101 | DD x CD | 06F9 | 24 |

PHOTOS OF TEST RESULTS

| a) P M Q | b) |
| --- | --- |
| c) | d) |
| e) | f) |