Name: _____Servando_Olvera________ ID# ________1001909287_______ Date Submitted: _____04-27-2024____ Time Submitted _____4:00_pm______ CSE 3341 Digital Logic Design II CSE 5357 Advanced Digital Logic Design Spring Semester 2024 Eight-Bit, Four Function Calculator 250+ points Due Date – April 30, 2024, 11:59 PM Submit on Canvas Assignments

DESIGN REQUIREMENTS

PURPOSE/OUTCOMES
To design, implement on the DE10-Lite + KeyPad + HexBoard, and test an eight-bit, four-function (ADD, SUBTRACT, MULTIPLY, DIVIDE) calculator. The calculator can be partitioned in to four components as illustrated in Figure 1. You will perform this project by designing the Control Unit (CU) and integrating it with the Arithmetic Unit (AU), Output Unit (OU), and Input Unit (IU) designed in previous assignments. By successfully completing this project, you will have demonstrated an ability to design, implement, and test a machine that incorporates combinational and sequential logic circuits implemented with field programmable logic arrays (FPGAs) and designed with the SystemVerilog hardware description language (HDL).

REQUIREMENTS
To successfully complete the term project, you must design the CU and integrate it with the IU, AU, and OU and demonstrate it’s functionality with the test inputs specified below.

a. 74 + 35
b. 74 – 35
c. –74 – 35
d. 127 + 6
e. 10 x 15
f. 127 x 2
g. –1 x –1
h. –127 x –127
i. 10 ÷ 5
j. 67 ÷ 32

INPUT REQUIREMENTS
1. Operands must be entered in hexadecimal on the CSE KeyPad 4 x 4 keypad shown in Figure 2.
2. Control and operations must be entered using pushbuttons as described below.

NUMBER ENTRY AND OPERATIONS REQUIREMENTS
Inputs consist of a sign and up to three magnitude digits. Enter operands and perform operations as follows.

1. Clear the calculator – press Key0 (Clear All)
2. Capture operand A using the KeyPad
3. Enter operand Ahigh – press Key1
4. Enter operand Alow – press Key1
5. Capture operand B using the KeyPad
6. Enter the operand B – press Key1
7. Enter the operation to be performed – add (Key2), subtract (Key3), multiply (Key4), divide (Key5)

DISPLAY REQUIREMENTS
1. Operands and results must be displayed in hexadecimal on the HexBoard.
2. Results must be displayed in decimal sign-magnitude on the DE10 seven-segment displays after
calculations are complete.
3. Operations that produce an oveflow must light LEDR9.
4. Operations that produce a zero (0) result must light LEDR8.

CONTROL UNIT REQUIREMENTS
The control-path, data-path interface is shown in Figure 3. The control signals are produced by a finite state machine described in Figure 4.

DESCRIPTION OF ADDED FEATURES

N/A

No added features.

ORGANIZATION DIAGRAM

CONTROL UNIT

Inputs/Output

State Diagram

TEST RESULTS

Summary

| Test Case | Inputs (Decimal) | Expected Result (Decimal) | Expected Result (Hex) | Actual Inputs (Hex) | Actual Result | OVR | ZERO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a. | 74 + 35 | 109 | 006D | 4A + 23 | 006D | 0 | 0 |
| b. | 74 - 35 | 39 | 0027 | 4A - 23 | 0027 | 0 | 0 |
| c. | -74 - 35 | -109 | FF93 | B6 - 23 | FF93 | 0 | 0 |
| d. | 127 + 6 | 133 | 0085 | 7F + 06 | FF85 | 1 | 0 |
| e. | 10 x 15 | 150 | 0096 | 0A x 0F | 0096 | 0 | 0 |
| f. | 127 x 2 | 254 | 00FE | 7F x 02 | 00FE | 0 | 0 |
| g. | -1 x -1 | 1 | 0001 | FF x FF | 0001 | 0 | 0 |
| h. | -127 x -127 | 16,129 | 3F01 | 81 x 81 | 3F01 | 0 | 0 |
| i. | 10 ÷ 5 | 2¦0 | 02¦00 | 0A ÷ 05 | 02¦00 | 0 | 0 |
| j. | 67 ÷ 32 | 2¦3 | 02¦03 | 43 ÷ 20 | 02¦03 | 0 | 0 |

Demonstration (Documented Pictures)

| Test Case | Input A | Input B | Op | Result |
| --- | --- | --- | --- | --- |
| a. |  |  | + |  |
| b. |  |  | - |  |
| c. |  |  | - |  |
| d. |  |  | - |  |
| e. |  |  | + |  |
| f. |  |  | x |  |
| g. |  |  | x |  |
| h. |  |  | x |  |
| i. |  |  | ÷ |  |
| j. |  |  | ÷ |  |

VERILOG CODE

Term Project/ Top Module

Control Unit Module

MUX

Arithmetic Unit Module

Arithmetic Unit Module

Clock Ladder Modified Module

Edge Detect Module